## # $ K Structure of the Pipeline

The processor "DLX" (read: DeLuXe) has a pipeline with five pipe stages:

+ IF-Stageh_IF_Stage

+ ID-Stageh_ID_Stage

+ EX-Stages:h_EX_Stage

- intEX-Stageh_intEX_Stage

- faddEX-Stage(s)h_faddEX_Stage

- fmulEX-Stage(s)h_fmulEX_Stage

- fdivEX-Stage(s)h_fdivEX_Stage

+ MEM-Stageh_MEM_Stage

+ WB-Stageh_WB_Stage

## # $ K IF-Stage (Instruction Fetch)

On each clock cycle another instruction is fetched from memory into the instruction register. The PCh_PC is incremented to point to the next instruction.

## # $ K ID-Stage (Instruction decoding)

The instruction is decoded and the register file is accessed to read the operand registers Ah_A_B and Bh_A_B. Branches are already executed in this stage to reduce the control stalls.

## # $ K EX-Stages (EXecution)

In this stage, the proper unit is selected to operate on the operands prepared in the prior step. The execution stage contains units for integer operations (intEXh_intEX_Stage exists once) and for floating point operations: faddEXh_faddEX_Stage, fmulEXh_fmulEX_Stage and fdivEXh_fdivEX_Stage (multiple units possible).

## # $ K intEX-Stage (INTeger EXecution):

This unit performs integer arithmetic operations (except multiply and divide) and effective address calculation for memory references and branches.

## # $ K faddEX-Stage (Floating point ADDition EXecution):

This unit performs floating point addition and subtraction of single- or double precision floating point numbers.

## # $ K fmulEX-Stage (Floating Point MULtiplication EXecution):

This unit performs multiply operations of single or double precision floating point numbers but also on integer numbers (signed and unsigned).

## # $ K fdivEX-Stage (Floating Point DIVision EXecution):

This unit performs divide operations of single or double precision floating point numbers but also on integer numbers (signed and unsigned).

## # $ K MEM-Stage (MEMory access)

The only DLX instructions active in this step are loads and stores. If the instruction is a load, data are read from memory; if it is a store, then the data are written into memory. In either case the address used is the one computed during the prior step.

## # $ K WB-Stage (Write Back)

The result is written into the register file, either coming from the memory system or from the ALU. The operations in this stage are performed during the first half of the clock cycle, so the instruction executing the ID-stage can read the results in the second half of the clock cycle, eliminating the need to bypass the reult to this instruction.

## # $ K Register Set

DLX has the following registers, that are visible to programs:

+ GPRs (R0..R31)h_GPRs

+ FPRs (F0..F31)h_FPRs

+ FPSRh_FPSR

+ PCh_PC

DLX has also internal registers, that are not visible to DLX-programs:

+ IMARh_IMAR

+ IRh_IR

+ A, Bh_A_B

+ BTAh_BTA

+ ALUh_ALU

+ DMARh_DMAR

+ SDRh_SDR

+ LDRh_LDR

## # $ K General Purpose Register (GPRs, Integer Register)

The architecture has thirty-two 32-bit general-purpose registers; the value of R0 is always 0.

## # $ K Floating Point Registers (FPRs)

There are a set of Floating point registers, which can be used as 32 single-precision (32-bit) registers (F0 to F31), or as even-odd pairs holding double-precision values (D0 to D30).

## # $ K Floating Point Status Register (FPSR)

The FP status register is used both for compares and FP exceptions. All moves to/from the status register go through the GPRs; there is a branch that tests the comparison bit in the FP status register.

## # $ K Program Counter (PC)

The program counter always contains the address of the next instruction to be fetched. Branches and jumps can change its contents.

## # $ K Instruction Memory Address Register (IMAR)

This register is initialized with the content of the program counter in the IF stage, because it has a connection to the memory system, but PC doesn't.

## # $ K Instruction-Register (IR)

In the IF-stage, this register is loaded with the next instruction to be executed.

## # $ K ALU Operand Registers A and B

In the ID-stage, the two source registers are read and their value is sent to the ALU operand registers A and B. In WinDLX, there also exist the (Pseudo-)registers AHI und BHI, that contain the upper 32 bits for double-precision Floating point values.

## # $ K Branch Target Address Register (BTA)

In the ID-stage, the branch/jump target address is computed and written into this register. If the branch is taken, this value is loaded to the program counter.

## # $ K ALU Output Register (ALU)

The result of an ALU operation is transferred to this register. In WinDLX, there exists the (pseudo-)register ALUHI, that contains the higher 32 bits for double-precision Floating point values.

## # $ K Data Memory Address Register (DMAR)

The address for a memory reference is transferred to this register. In the MEM-stage, the memory reference is performed with this register.

## # $ K Store Data Register (SDR)

For a store instruction, the data to be stored into the memory is transferred to this register. In WinDLX, there exists the (pseudo-)register SDRHI, that contains the higher 32 bits of double-precision Floating point values.

## # $ K Load Data Register (LDR)

For a load instruction, the data is read from memory into this register. In WinDLX, there exists the (pseudo-)register LDRHI, that contains the higher 32 bits of double-precision Floating point values.

## # $ K Instruction Set of DLX

There are four classes of instructions:

+ Data transfer instructionsh_Datatransfer_instructions

+ Integer arithmetic and logical instructionsh_integer_instructions

+ Control instructionsh_Control_instructions

+ Floating point instructionsh_Floatingpoint_instructions

All instructions are 32 bits wide with a 6-bit primary opcode. Basically, there are three different instruction types:

I-type instructions:

Encodes:

+ Loads- and Stores (rd <- Mem[rs1+Immediate])

+ All immediates (rd <- rs1 op Immediate)

+ Conditional branch instructions (rs1 is register, rd unused)

+ JR (jump register) and JALR (jump and link register), where rd=0, rs1=Register, Immediate=0

R-type instructions:

+ Register-Register ALU operations rd <- rs1 func rs2:

+ MOVxxx-Instructions (rd <- rs1)

J-type instructions:

Encodes:

+ J (jump) and JAL (jump and link)

+ TRAP

+ RFE (return from exception)

## # $ K Instructions for Data Transfer

Move data between registers and memory, or between integer and FP or special registers; only memory address mode is 16-bit displacement + contents of a GPR:

LB Rd,Adr	Load byte (sign extension)

LBU Rd,Adr	Load byte (unsigned)

LH Rd,Adr	Load halfword (sign extension)

LHU Rd,Adr	Load halfword (unsigned)

LW Rd,Adr	Load word

LF Fd,Adr	Load single-precision Floating point

LD Dd,Adr	Load double-precision Floating point

SB Adr,Rs	Store byte

SH Adr,Rs	Store halfword

SW Adr,Rs	Store word

SF Adr,Fs	Store single-precision Floating point

SD Adr,Fs	Store double-precision Floating point

MOVI2FP Fd,Rs	Move 32 bits from integer registers to FP registers

MOVI2FP Rd,Fs	Move 32 bits from FP registers to integer registers

MOVF Fd,Fs	Copy one Floating point register to another register

MOVD Dd,Ds	Copy a double-precision pair to another pair

MOVI2S SR,Rs	Copy a register to a special register (not implemented!)

MOVS2I Rs,SR	Copy a special register to a GPR (not implemented!)

## # $ K Integer arithmetic and logical instructions

Operations on integer or logical data in GPRs; overflows by signed arithmetics are not reported.

ADD Rd,Ra,Rb	Add

ADDI Rd,Ra,Imm	Add immediate (all immediates are 16 bits)

ADDU Rd,Ra,Rb	Add unsigned

ADDUI Rd,Ra,Imm	Add unsigned immediate

SUB Rd,Ra,Rb	Subtract

SUBI Rd,Ra,Imm	Subtract immediate

SUBU Rd,Ra,Rb	Subtract unsigned

SUBUI Rd,Ra,Imm	Subtract unsigned immediate

MULT Rd,Ra,Rb	Multiply signed

MULTU Rd,Ra,Rb	Multiply unsigned

DIV Rd,Ra,Rb	Divide signed

DIVU Rd,Ra,Rb	Divide unsigned

AND Rd,Ra,Rb	And

ANDI Rd,Ra,Imm	And immediate

OR Rd,Ra,Rb	Or

ORI Rd,Ra,Imm	Or immediate

XOR Rd,Ra,Rb	Xor

XORI Rd,Ra,Imm	Xor immediate

LHI Rd,Imm	LOad high immediate - loads upper half of register with immediate

SLL Rd,Rs,Rc	Shift left logical

SRL Rd,Rs,Rc	Shift right logical

SRA Rd,Rs,Rc	Shift right arithmetic

SLLI Rd,Rs,Imm	Shift left logical 'immediate' bits

SRLI Rd,Rs,Imm	Shift right logical 'immediate' bits

SRAI Rd,Rs,Imm	Shift right arithmetic 'immediate' bits

S__ Rd,Ra,Rb	Set conditional: "__" may be EQ, NE, LT, GT, LE or GE

S__I Rd,Ra,Imm	Set conditional immediate: "__" may be EQ, NE, LT, GT, LE or GE

S__U Rd,Ra,Rb	Set conditional unsigned: "__" may be EQ, NE, LT, GT, LE or GE

S__UI Rd,Ra,Imm	Set conditional unsigned immediate: "__" may be EQ, NE, LT, GT, LE or GE

NOP	No operation

## # $ K Control instructions

Control is handled through a set of jumps and a set of branches. the four jump instructions are differentiated by the two ways of specifying the destination address and by wether or not a link is made. Two jumps use a 26-bit offset added to the program counter to determine the destination address; the other two specify a register that is the destination address. There are two flavors of jumps: plain jump, and jump and link (used for procedure call). The latter places the return address in R31. All branches are conditional. The branch condition is specified by the instruction, which may test the register source for zero or nonzero; this may be a data value or the result of a compare. The branch target address is specified with a 16-bit signed offset that is added to the program counter. In WINDLX, the jumps and the branches are completed by the end of the ID cycle to reduce the number of control stalls. In the DLX pipeline, the predict-not-taken scheme is implemented by continuing to fetch instructions as if the branch were a normal instruction. The pipeline looks as if nothing out of the ordinary is happening. If the branch is taken, we neeed to stop the pipeline and restart the fetch.

BEQZ Rt,Dest	Branch if GPR equal to zero; 16-bit offset from PC

BNEZ Rt,Dest	Branch if GPR not equal to zero; 16-bit offset from PC

BFPT Dest	Test comparison bit in the FP status register (true) and branch; 16-bit offset from PC

BFPF Dest	Test comparison bit in the FP status register (false) and branch; 16-bit offset from PC

J Dest	Jump: 26-bit offset from PC

JR Rx	Jump: target in register

JAL Dest	Jump and link: save PC+4 to R31; target is PC-relative

JALR Rx	Jump and link: save PC+4 to R31; target is a register

TRAP Imm	Transfer to operating system at a vectored address; see Trapsh_Traps.

RFE Dest	Return to user code from an execption; restore user mode (not implemented!)

## # $ K Floating Point Instructions

Floating point instructions manipulate the floating point registers and indicate whether the operation to be performed is single or double precision. Single-precision operations can be applied to any of the registers, while double-precision operations apply only to even-odd pair (e.g. F4,F5), which is designated by the even register number.

ADDD Dd,Da,Db	Add double-precision numbers

ADDF Fd,Fa,Fb	Add single-precision numbers

SUBD Dd,Da,Db	Subtract double-precision numbers

SUBF Fd,Fa,Fb	Subtract single-precision numbers.

MULTD Dd,Da,Db	Multiply double-precision Floating point numbers

MULTF Fd,Fa,Fb	Multiply single-precision Floating point numbers

DIVD Dd,Da,Db	Divide double-precision Floating point numbers

DIVF Fd,Fa,Fb	Divide single-precision Floating point numbers

CVTF2D Dd,Fs	Converts from type single-precision to type double-precision

CVTD2F Fd,Ds	Converts from type double-precision to type single-precision

CVTF2I Fd,Fs	Converts from type single-precision to type integer

CVTI2F Fd,Fs	Converts from type integer to type single-precision

CVTD2I Fd,Ds	Converts from type double-precision to type integer

CVTI2D Dd,Fs	Converts from type integer to type double-precision

__D Da,Db	Double-precision compares: "__" may be EQ, NE, LT, GT, LE or GE; sets comparison bit in FP status register

__F Fa,Fb	Single-precision compares: "__" may be EQ, NE, LT, GT, LE or GE; sets comparison bit in FP status register

## # $ K Traps - the System Interface

Traps build the interface between DLX programs and the I/O-system. There are five traps defined in WinDLX. Zero is an invalid parameter for a trap instruction, used to terminate a program.

The Traps:

+ Trap #1: Open Fileh_Trap1

+ Trap #2: Close Fileh_Trap2

+ Trap #3: Read Block from Fileh_Trap3

+ Trap #4: Write Block to Fileh_Trap4

+ Trap #5: Formatted Output to Standard-Outputh_Trap5

For all five defined traps:

+	They match the UNIX/DOS-System calls resp. C-library-functions open(), close(), read(), write() and printf().

+	The file descriptors 0,1 and 2 are reserved for the stdin, stdout and stderr. The input and output to the DLX-I/O-Windows can be controlled with this descriptors.

+	The address of the required parameters for the system calls must be loaded in register R14.

+	All parameters have to be 32 bits long (except the double-precision FPs that are 64 bits long). Strings are referenced with their begin address.

+	The result is returned into R1.

+	If an error occurs during the execution of a system call, register R1 is set to -1 and: if the symbol "_errno" is set to value A, then an error code is returned at the memory address A and the simulation continues, otherwise the simulation is aborted. See MS-DOS documentation for details about the error codes.

## # $ K Trap #1: Open File

You can open a file for reading or to writing. Open files are automatically closed after a DLX reset or when quitting the WinDLX.

Parameter:

1. filename: Address of a zero-terminated string, that contains the path of the file to be opened.

2. mode, to open the file. Following modes can be combined using logical or-operation:

0x0001		O_RDONLY	(read only)

0x0002		O_WRONLY	(write only)

0x0004		O_RDWR	(read and write)

0x0100		O_CREATE	(create file)

0x0200		O_TRUNC	(truncate file)

0x0400		O_EXCL	(open file exclusively (with SHARE))

0x0800		O_APPEND	(append to file)

0x4000		O_TEXT	(Convert CR/LF)

0x8000		O_BINARY	(No conversion of CR/LF)

(Warning: These flags are predefined in MS-DOS and not compatible to UNIX.)

3. Additional Flags:

0x0000		S_IFREG	(Normal file, no directory etc.)

0x0100		S_IREAD	(read access permission)

0x0080		S_IWRITE	(write access permission)

0x0040		S_IEXEC	(execute permission)

(Warning: These flags are predefined in MS-DOS and not compatible to UNIX.)

(See C-Library functions for more details.)

The file descriptor is returned in register R1.

Example:

.data

FileName:	.asciiz	"c:\\BSP\\DATEI.DAT"

.align	2

Par:	;*** Parameters for Trap1 (OPEN)

.word	FileName

;create for R/W:

.word	0x0104

;R/W-access permission:

.word	0x0180

FileDescr:	.space	4

.text

lhi	r14,Par>>16		; upper 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	1

sw	FileDescr,R1

## # $ K Trap #2: Close File

A file previously opened with Trap #1 is closed.

Parameter:

1. File descriptor of the file to be closed.

Zero is returned in register R1, if it was successful, -1 otherwise.

Example:

.data

FileDescr:	.space	4

.text

;...

lhi	r14,FileDescr>>16		; higher 16 bits

addui	r14,r14,FileDescr&0xffff	; lower 16 bits

trap	2

## # $ K Trap #3: Read Block from File

A file block or a line from stdin can be read with this trap.

Parameter:

1. File descriptor of the file

2. Address, for the destination of the read operation

3. Size of block (bytes) to be read

The actual number of bytes read is returned in register R1.

Example:

.data

Buffer:	.space	500

Par:	;File descriptor, Adr. of buffer, size of block

.space	4

.word	Buffer

.word	500

.text

;...

lhi	r14,Par>>16		; higher 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	3

## # $ K Trap #4: Write Block to File

A block can be written to the memory or the standard output.

Parameter:

1. File descriptor of file

2. Address of block to be written

3. Size of the block (bytes)

The actual number of bytes written is returned in register R1.

Example:

.data

Buffer:	.space	500

Par:	;File descriptor, Adr. of buffer, size of block

.space	4

.word	Buffer

.word	500

.text

;...

lhi	r14,Par>>16		; higher 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	4

## # $ K Trap #5: Formatted Output to Standard Output

This function is equivalent to the C-library function printf().

Parameter:

1. Format string see C-function printf()

2.... Arguments according to format string; see C-function printf()

The number of bytes transferred to stdout is returned in register R1.

Example:

.data

FormatStr:	.asciiz	"Pi=%f, N=%d\n"

.align	2

Par:	;Adr. of formatstring, values

.word	FormatStr

.double	3.141592654

.word	17

.text

;...

lhi	r14,Par>>16		; higher 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	5

## # $ K The DLX-Assembly language

In WinDLX, you can load one or more DLX-source files. The files are loaded in alphabetical order.. In addition to the DLX-Instructionsh_Instructions , the assembler supports several directives which affect how it fills the memory.

+ Syntax for Expressionsh_Syntax

+ Directivesh_directives

## # $ K Syntax for Expressions

The syntax for expressions is C-like (e.g. strings must be quoted with " ").

Numbers: WinDLX accepts numbers in decimal notation, hexadecimal notation if the first two characters of the number are 0x, or octal notation if the first character is 0.

Address expressions: The simplest form of such an expression is a number, which is interpreted as a memory address. More generally, address expressions may consist of numbers, symbols (which must be defined in the assembly files currently loaded) , the operators *, /, +, -, <<, >>, &, |, and ^ (which have the same meanings and precedences as in C), and parentheses for grouping.

The value of an expression must be in the defined range proper to the type of the expression:

Bytes:		-128 .. +255 (8 Bit)

Halfwords:	-32768 .. +65535 (16-Bit)

Words:		-2147483648 ..+2147483647 oder 0xffffffff (32-Bit)

## # $ K Directives

While the assembler is processing an assembly file, the data and instructions it assembles are placed in memory based on either a text (code) or data pointer. Which pointer is used is not selected by the type of information, but whether the most recent directive was .data or .text. The program initially loads into the text segment. By default, the code is loaded at location $CODE (initially set to 0x100) and the data are loaded at location $DATA (initially set to 0x1000).

.align n	Cause the next data/code loaded to be at the next higher address with the lower n bits zeroed (the next closest address greater than or equal to the current address that is a multiple of 2n (e.g. .align 2 means the next word begin)

.ascii "string1","..."	Store the "strings" listed on the line in memory as a list of characters. The strings are not terminated by a 0 byte.

.asciiz "string1","..."	Similar to .ascii, except each string is terminated by a 0 byte.

.byte byte1,byte2,...	Store the bytes listed on the line sequentially in memory.

.data [address]	Cause the following code and data to be stored in the data area. If an address was supplied, the data will be loaded starting at that address, otherwise, the last value for the data pointer will be used. If we were just reading data based on the text (code) pointer, store that address so that we can continue from there later (on a .text directive).

.double number1,...	Store the "numbers" listed on the line sequentially in memory as double-precision Floating point numbers.

.global label	Make the label available for reference by code found in files loaded after this file.

.space size	Move the current storage pointer forward size bytes (to leave some empty space in memory)

.text [address]	Cause the following code and data to be stored in the text (code) area. If an address was supplied, the data will be loaded starting at that address, otherwise, the last value for the text pointer will be used. If we were just reading data based on the data pointer, store that address so that we can continue from there later (on a .data directive).

.word word1,word2,...	Store the word" listed on the line sequentially in memory.

## # $ K Execution of Instructions

For the DLX instruction set (excluding the Execution of Floating point instructionsh_Floatingpoint_execution), all instructions can be broken into five basic steps: fetch, decode, execute, memory-access, and write-result. This steps match the five stages of the DLX pipeline. The following table shows the events on every pipe stage of the DLX pipeline:

Stage	ALU instruction	Load or store instruction	Branch instruction

IF	IMAR <- PC;	IMAR <- PC;	IMAR <- PC;

IR <- Mem[IMAR]	IR <- Mem[IMAR]	IR <- Mem[IMAR]

PC <- PC + 4	PC <- PC + 4	PC <- PC + 4

ID	A <- Rs1; B <- Rs2;	A <- Rs1; B <- Rs2;	A <- Rs1; B <- Rs2;

BTA <- PC + offset;

if (Rs1 op 0) then PC <- BTA;

EX	ALUoutput<- A op B; or	DMAR <- A + immediate;

ALUoutput <- A op immediate	SDR <- B;

MEM	LDR <- Mem[DMAR]; or

Mem[DMAR] <- SDR

WB	Rd <- ALUoutput	Rd <- LDR

The major hurdle of pipelining - Pipeline Hazards

There are situations, called hazards, that prevent the next instruction in the instruction stream from being executed during its designated clock cycle. Hazards reduce the performance of the ideal speedup gained by pipelining. Hazards in pipelines can make it necessary to stall the pipeline. The impact of hazards in causing stalls can be reduced by the use of forwardingh_forwarding. There are three classes of hazards:

+ Structural hazardsh_Structural_hazards

+ Data hazardsh_Data_hazards

+ Control hazardsh_Control_hazards

In WinDLX, traps can also can cause stalls, see Trap instructionh_Trap_instructions.

## # $ K Structural Hazards

Structural hazards arise from resource conflicts when the hardware cannot support all possible combinations of instructions in simultaneous overlapped execution.

## # $ K Data Hazards

Data hazards arise when an instruction depends on the results of a previous instruction in a way that is exposed by the overlapping of instructions in the pipeline. Data hazards may be classified as one of three types, depending on the order of read and write accesses in the instructions. Consider two instructions i and j, with i occuring before j. The possible data hazards are:

i) RAW (read after write): j tries to read a source before i writes it, so j incorrectly gets the old value.

ii) WAR-(write after read): j tries to write a destination before it is read by i, so i incorrectly gets the new value. This cannot happen in WinDLX because all reads are early (in ID) and all writes are late (in WB). This hazard occurs when there are some instructions that write results early in the instruction pipeline, and other instructions that read a source after a write of an instruction later in the pipeline (e.g. autoincrement addressing).

iii) WAW (write after write): j tries to write an operand before it is written by i. The writes end up being performed in the wrong order, leaving the value written by i rather than the value written by j in the destination. This hazard is present only in pipelines that write in more than one pipe stage. The DLX pipeline writes a register only in WB and avoids this class of hazards for integer instructions. This hazard can occur with floating point stage instructions, see Execution of Floating Point Instructionsh_Floatingpoint_execution

## # $ K Control Hazards

Control hazards arise from the pipelining of branches and other instructions that change the PC. If instruction i is a taken branch, then the PC is normally not changed until the end of ID.

## # $ K Forwarding

Forwarding (also called bypassing and sometimes short-circuiting) is a simple hardware technique to reduce the penalties caused by data hazards. This technique works as follows: The ALU result is always fed back to the ALU input latches. If the forwarding hardware detects that the previous ALU operation has written the register corresponding to a source for the current ALU operation, control logic selects the forwarded result as the ALU input rather than the value read from the register file.

## # $ K Trap Instructions

In WinDLX, traps are delayed using stalls until no prior instructions are in the pipeline. That is performed to avoid conflict with other instructions.

## # $ K Execution of Floating Point Instructions

The following DLX instructions use the Floating point execution stages:

ADDF, ADDD, SUBF, SUBD	faddEX

MULT, MULTU, MULTF, MULTD	fmulEX

DIV, DIVU, DIVF, DIVD	fdivEX

The Floating point execution stages take more than one cycle to perform the operation (The number of FP units and their delay can be configured in WinDLX).

If more than one Floating point instruction terminate their execution stage simultaneously, it is not defined which instruction enters the MEM stage first. If an integer instruction and a Floating point instruction terminate their execution stage simultaneously, the Floating point instruction enters the next pipeline stage first.

WAW hazards (see Data hazardsh_Data_hazards) can occur with the Floating point instructions (do not complete in order). This conflict is detected in WinDLX before the instruction enters the EX stage and causes WAW stalls.

A simple example:

ADDF	F1,F2,F3	IF	ID	EX	EX	MEM	WB

MOVF	F1,F4			IF	ID	Stall	EX	MEM	WB

# h_Pipelinestructure

$ DLX-Pipeline structure

K Pipeline; Pipeline structure

# h_IF_Stage

$ IF Stage

K IF Stage; Pipeline

# h_ID_Stage

$ ID-Stage

K ID-Stage; Pipeline

# h_EX_Stage

$ EX-Stage

K EX-Stage; Pipeline

# h_intEX_Stage

$ intEX-Stage

K intEX-Stage; Pipeline

# h_faddEX_Stage

$ faddEX-Stage

K faddEX-Stage; Pipeline; Floatingpoint stages

# h_fmulEX_Stage

$ fmulEX-Stage

K fmulEX-Stage; Pipeline; floating point stages

# h_fdivEX_Stage

$ fdivEX-Stage

K fdivEX-Stage; Pipeline; Floating point stages

# h_MEM_Stage

$ MEM-Stage

K MEM-Stage; Pipeline

# h_WB_Stage

$ WB-Stage

K WB-Stage; Pipeline

# h_Registerset

$ DLX-Register set

K Register; Register set

# h_GPRs

$ General-Purpose-Register (GPRs)

K GPRs; Integer-Register

# h_FPRs

$ Floating-point Registers (FPRs)

K FPRs; Floating-point registers

# h_FPSR

$ Floating-point Statusregister (FPSR)

K FPSR; Statusregister

# h_PC

$ Program-Counter (PC)

K PC

# h_IMAR

$ Instruction-Memory-Address-Register (IMAR)

K IMAR

# h_IR

$ Instruction-Register (IR)

K IR

# h_A_B

$ ALU-Operands-Registers A and B

K A; B

# h_BTA

$ Branch-Target-Address-Register (BTA)

K BTA

# h_ALU

$ ALU-Output-Register (ALU)

K ALU

# h_DMAR

$ Data-Memory-Address-Register (DMAR)

K DMAR

# h_SDR

$ Store-Data-Register (SDR)

K SDR

# h_LDR

$ Load-Data-Register (LDR)

K LDR

# h_Instructions

$ Instructions of DLX

K Instruction; Instruction set

# h_Datatransfer_instructions

$ Data transfer instructions

K Instruction; Instruction set

# h_Integer_instructions

$ Integer arithmetic and logical instructions

K Instruction; Instruction set

# h_Control_instructions

$ Control instructions

K Instruction; Instruxtion set; Branches

# h_Floatingpoint_instructions

$ Floating point instructions

K Instruction; Instruction set; Floating point instructions

# h_Traps

$ Traps - the System Interface

K Traps

# h_Trap1

$ Trap #1: Open File

K Traps

# h_Trap2

$ Trap #2: Close File

K Traps

# h_Trap3

$ Trap #3: Read Block from File

K Traps

# h_Trap4

$ Trap #4: Write Block to File

K Traps

# h_Trap5

$ Trap #5: Formatted output to the Standard Output

K Traps

# h_DLX_Assembly_Language

$ DLX-Assembly language

K Assembler; Disassembler; Instruction

# h_Syntax

$ Syntax for Expressions

K Expressions; Syntax

# h_directives

$ Directives

K Data; Assembler

# h_Instruction_execution

$ execution of instructions

K instruction; execution; hazards; simulation; stalls

# h_Structural_hazards

$ Struatural hazards

K Stalls; hazards; structural stalls; structural hazards

# h_Data_hazards

$ Data hazards

K Stalls; Data hazards; RAW stalls; WAW stalls; WAR stalls

# h_Control_hazards

$ Control hazards

K Stalls; Control hazards

# h_forwarding

$ Forwarding

K Forwarding, bypassing

# h_Trap_instructions

$ Trap instructions

K Stalls; Trap stalls; Traps

# h_Floatingpoint_execution

$ Floating point instructions

K Floating point instructions; execution