## WinDLX V1.2

A DLX-Simulator for MS-Windows

January 1992  Vienna University of Technology

Contact address:	Vienna University of Technology

Prof. Herbert Grünbacher

Dept. for VLSI-Design

Treitlstr. 3/2

A-1040 Vienna

FAX: (+43-1) 56 96 97

Help index:

+ General Concepts and Child Windowsh_Concepts

+ Main menuh_Mainmenu

+ The Keyboardh_Keyboard

+ The DLX Pipeline Structureh_Pipelinestructure

+ The DLX Register Seth_Registerset

+ The DLX-Assembly Languageh_DLX_Assembly_Language

+ The DLX-Instructionsh_Instructions

+ Traps - The System Interfaceh_Traps

+ Execution of  Instructions with Stalls and Forwardingh_Instruction_execution

+ DLX-Examplesh_DLX_Examples

## System Requirements and Installation

WINDLX will run on IBM-AT-compatible PCs with 80286, 80386 and 80486-processors with at least 1MB main memory and VGA graphics display, provided that the operating system MS-DOS 3.3 (or a newer version) with the graphical interface MS-Windows 3.0 (standard- or enhanced mode) are installed. A mouse is not required but very useful.

The installation of WINDLX is very simple. You only copy the files WINDLX.EXE and WINDLX.HLP from the floppy to an arbitrary directory on the harddisk (perferably C:\WINDLX). WINDLX can also be started directly from the floppy, accepting longer response times during loading and execution.

WINDLX.EXE runs under MS-Windows like any other Windows application.

## General Concepts of WINDLX

In the Main window (also called Frame window) are several iconized Child windows, which - if redisplayed - display different pieces of information:

+ The Register Windowh_Registerwindow (Register contents)

+ The Code Windowh_Codewindow (DLX-Instructions in Memory)

+ The Pipeline Windowh_Pipelinewindow (Pipeline stages with contained instructions)

+ The Clock Cycle Diagramh_Clockcycle_diagram (What is done in which cycle in which stage?)

+ The Statistics Windowh_Statisticwindow (Statistics about the execution of instructions)

+ The Breakpoint Windowh_Breakpointwindow (Set/Delete/View Breakpoints)

+ Up to 10 Memory Windowsh_Memorywindow (Display Memory contents)

The size of the Child windows is limited by the size of the Frame window. They can't be closed (The only exception is the dynamically created Memory window). Before using WINDLX, you should become familiar with windows and how to operate within them.

Across the top of the Frame window and below the title bar is a list of menu titles, the Main menuh_Mainmenu. The second rightmost title represents the currently active Child window. Each Child window itself has only a system menu, with options for resizing, moving, closing or iconizing the window.

## The Main Menu

The Frame menu has following options available, which on their part also have submenus:

+ Fileh_File

+ Windowh_Window

+ Executionh_Execution

+ Memoryh_Memory

+ Configurationh_Configuration

+ Helph_Help

Furthermore, the second rightmost title is the currently active Childwindow.

## File

Following options are available:

+ Reset DLXh_DLX_reset

+ Reset Allh_Reset_All

+ Load Code or Datah_Load_Code_or_Data

+ Quit WINDLXh_Quit_WINDLX

## Reset DLX

Selecting this option performs the following operations:

+ The pipeline will be cleared.

+ The statistics will be reset.

+ All registers will be initialized (PC with the value $TEXT, all other with zero).

+ The whole history information will be deleted.

+ All opened files will be closed.

+ DLX-I/O-redirection will be canceled.

+ The content of the DLX-I/O-Window will be deleted.

The content of memory and all defined symbols remain unchanged.

## Reset All

This option will perform all the activities of "Reset DLX"h_DLX_Reset In addition, the memory will be initialized with zero and the symbols will be deleted. The symbols $TEXT and $DATA remain unchanged (saved in configurations).

## Load Code or Data

If you select this option, a dialog box will appear. You can select an arbitrary number of DLX-source code modules in files with the extension ".S". If you confirm the selection with the Load-push button, all selected modules will be assembled and the code is written to the DLX-Memory. error-reports will appear in a separate dialog box and the data written to the memory are invalid. After a successful loading, you have the possibility to reset the DLX-Processor via dialog box.

If you have selected the wrong file or change your mind, select its name and press the Delete-Button. If you don't select any files, no files will be loaded.

The possibility to load multiple modules at the same time allows to define global symbols that can be used in multiple modules. Furthermore, it is possible to load modules regardless of their order.

## Quit WINDLX

If you select this option, you will be asked to save the current configuration, confirm the Quit or cancel it.

## Window

The Window-Submenu is the same as for all similar MS-Windows applications. There are several functions to automatically rearrange or select the Child windows:

+ Cascadeh_Cascade

+ Tileh_Tile

+ Arrange Iconsh_Arrange_Icons

+ (Child window in Window menu)h_Childwindowname_in_Windowmenu

## Cascade

This option displays all non-iconized Child windows so they all fit on the display at one time.

## Tile

This option displays all non-iconized Child windows so that each succeeding window overlaps the previous, leaving the upper left hand corner showing.

## Arrange Icons

All iconized Child windows will be automatically arranged across the bottom of the Frame window.

## Child Window Name in Window Menu

In the Window-Submenu exists also a list of all Child windows, which makes it possible to select them via menu (not only via double-clicking of the mouse button).

## Execution

In this menu are several commands to execute instructions and display the I/O-Window:

+ Single Cycleh_Single_Cycle

+ Multiple Cyclesh_Multiple_Cycles

+ Runh_Run

+ Run to...h_Run_to

+ Display DLX-I/Oh_Display_DLX_IO

## Single Cycle

A single cycle of the DLX processor will be simulated. The DLX-I/O-Windowh_DLX_IO_Window will only be displayed, when an In- or Ouput has occured in this cycle. If an error happens or an undefined trap during the simulation or a breakpoint is reached, a message window displays with detailed information about the state and the instruction causing the message.

## Multiple Cycles

After selecting this option, you can fix the number of cycles to be simulated in a dialog window. The last fixed value is the default value. Similar to the Runh_Run or Run to...h_Run_to options, the DLX-I/O-Windowh_DLX_IO_Window is displayed and you can abort the simulation via button every time.

The simulation will be aborted, when

+ a fixed number of cycles is simulated,

+ the user has pushed the Cancel button in the DLX-I/O-Window,

+ an error occured,

+ a breakpoint is reached,

+ an undefined trap is to process.

For the last three cases, a message window is displayed.

## Run

If you select this option, the simulation of the DLX instructions continues until

+ You stop the simulation via "Cancel"-button in the DLX-I/O-Windowh_DLX_IO_Window,

+ An error occurs,

+ A breakpoint is reached,

+ An undefined trap is to be processed.

In all these cases, a message window is displayed.

## Run to

Similar to Runh_Run, except the possibility to set a temporary brakpoint in a dialog boxh_Set. This temporary breakpoint is valid only for this run of the simulation, but it is saved as the default for the next time you activate this option.

## Display DLX-I/O

Activating this menu option displays the DLX-I/O-Windowh_DLX_IO_Window when the simulation is not running. All the options (e.g. redirecting the standard I/O etc.) can be performed.

## Memory

There are options to create Memory-Windows, change the content of memory and manipulate the symbols:

+ Displayh_Display

+ Changeh_Change

+ Symbolsh_Symbols

## Display

A new Memory windowh_Memorywindow is created, displaying memory contents in various types and formats.

After selecting this option, a dialog box is displayed. You can select the type, the format and the beginning address of the memory section to be displayed:

Type	Format

Byte (8-Bit)	hexadecimal, decimal or characters

Halfword (16-Bit)	hexadecimal or decimal

Word (32-Bit)	hexadecimal or decimal

single Floating point (32-Bit)	decimal

double Floating point (64-Bit)	decimal

The address can be an arbitrary integer expression, also containing operators and symbols.

The selected options are saved and used as defaults for the next time you activate this menu option.

## Change

Selecting this option lets you change or view memory contents at a given address. You can select the type of data to be changed or viewed (Byte, Halfword, Word, single- or double-precision Floatingpoints, String or instruction).

After entering an expression for the selected type and pressing the "Set"-Button, the new value is written to the memory address specified in the address area.

Pressing the "Next"-Button displays the content of the next memory address immediately following the specified address. The address is incremented by the number of bytes representing the selected type. Pressing the "Previous"-Button, you can display the contents of the previous memory cells in the same way.

The specified values are saved as defaults for the next time when this option is activated.

## Symbols

This option lets you manipulate all symbols via a dialog window.

In the symbol list, all symbols are displayed, sorted by name or value. Following activities are provided:

+ Define new, global symbols: Enter the name and the value of the symbol (any integer expression) and press the "New"-Button.

+ Change the value of an existing symbol: Select the symbol in the list box, change the value and press the "Change"-Button.

+ Delete an existing symbol: Select the symbol in the listbox and press the "Delete"-Button.

You can exit the window by pressing "OK".

## Configuration

Virtually all features of WinDLX can be customized to your personal tastes:

+ Size of the WINDLX-Frame window

+ State, size and position of all the Child windows

+ Default values for most of the dialog windows

+ the configuration of the floating point stages

+ the DLX-memory size

+ Use symbolic adresses? (Yes/No)

+ Absolute cycle count? (Yes/No)

+ Enable forwarding? (Yes/No)

+ the symbols $TEXT and $DATA (Start address of code- and data)

+ all settings in the Child windows

The configuration of WinDLX is saved into a file and can be loaded every time. This file must have the extension ".WDC". The file "WINDLX.WDC" is used as the default configuration file.

Saving the configuration can be performed at the end of a session (you will be prompted to save the configuration) or by activating the "Save" option in the configuration menu.

Commands on the configuration menu lets you customize the features that have general effects for the simulation:

+ Floating point stagesh_Floatingpointstages

+ Memory sizeh_Memorysize

+ Symbolic Addressesh_Symbolic_Addresses

+ Absolute Cycle Counth_Absolute_Cycle_count

+ Enable Forwardingh_Enable_Forwarding

+ Loadh_Load

+ Saveh_Save

## Floating Point Stages

The number of flooatingpoint units (1..8) and their delays (1..50 cycles) can be specified. Changing these values resets the DLX-processor automatically!

## Memory size

This option lets you modify the size of the memory available to the DLX processor. Theoretically, the memory size can be between 512 bytes (0x200) and 16 Megabytes (0x1000000). In practice, the memory size is constrained by memory management and the configuration of MS-Windows.

## Symbolic Addresses

If you enable this option, the addresses are displayed as an expression: "symbol + offset". Otherwise, the addresses are represented as hexadecimal values.

## Absolute Cycle Count

Enabling this option, the clock cycles are counted absolutely, beginning with the cycle 0 (resetting the processor or the statistics). Otherwise, the clock cycles are counted relatively, i.e. the current cycle is the cycle 0 and the previous cycles are labeled with -1, -2, etc.

## Enable Forwarding

With this option you can enable or disable the Forwarding-Mechanismh_Instruction_Execution.

## Load

When selecting this option, a list box is displayed and you can select a configuration file. The WinDLX will be initialized with the new configuration values. The contents of the memory and the symbols remain unchanged.

## Save

A dialog box is displayed and the current configuration can be saved in a file with the extension ".WDC". The default value is "WINDLX.WDC". When you start WinDLX, the configuration saved in WINDLX.WDC is loaded autmatically.

## Help

The Help menu contains commands that allow you to access WinDLX's help system and get general information about WinDLX. You can get context sensitive help information at any time pressing F1.

The help-system contains the user manual and a description of the DLX processor and its instruction set and the DLX assembly language.

## The Registerwindow

The current values of all registers are displayed in this window. It is possible to change the contents of the registers.

The Register Menu

There are several options in this menu to fix the type of representing the contents and the type of registers to be displayed:

+ Hexadecimalh_Hexadecimal

+ Decimalh_Decimal

+ Display Special Registersh_Display_Special_Registers

+ Display Integer Registersh_Display_Integer_Registers

+ Display Floatingpoint Registersh_Display_Floatingpoint_Registers

+ Display Doubleprecision-Floatingpoints h_Display_Doubleprecision_Floatingpoint

+ Display All Registersh_Display_All_Registers

Change the contents of Registers

You can change the contents of all GPRs, FPRs, the PC and the FPSR, but not the contents of the special (Hardware-)registers. You can select a register doubleclicking its name with the mouse. A dialog box is displayed, allowing you to change the content of the selected register. Legal values for integer registers are integer expressions and for floating point registers constant values.

## Hexadecimal

If you select this menu option, the contents of integer registers are represented hexadecimally. Floating point registers are always represented decimally.

## Decimal

The contents of the integer registers are displayed decimally. Floatingpoint registers are always represented decimalyl.

## Display Special Registers

The following registers are the special registersh_Registerset: PC, IMAR, IR, A, AHI, B, BHI, BTA, ALU, ALUHI, FPSR, DMAR, SDR, SDRHI, LDR und LDRHI. See the processor documentation for more information about these registers.

## Display Integer Registers

If this option is activated, the registers R0 to R31 (GPRsh_GPRs) and their contents are displayed.

## Display Floatingpoint Registers

If this option is activated, the single precision floatingpoint registersh_FPRs F0 to F31 are displayed in the Register window.

## Display Double precision Floatingpoints

If this option is activated, the double-precision floatingpoint contents of the floating point registersh_FPRs (F0&F1=D0, F2&F3=D2,...) are diaplyed in the register window.

## Display All Registers

If you activate this option, all registersh_Registerset are displayed in the Register window.

## The Code Window

The DLX instructions loaded in the memory and their addresses are diplayed (hexadecimal and disassembled) in the Code window. Further, the set breakpoints for each instruction are marked with Bxx, where xx is the type of the breakpoint. If an instruction is currently executing in a pipeline stage, the characteristic colour of the stage is used as background colour for the instruction and it is also labled adequately.

You can scroll up/down the memory using the arrows or the page keys. After executing one or more steps of a DLX code, the instructions executed at last are displayed.

You can get detailed information about an instruction being in the pipeline or set /delete breakpoints selecting the instruction with the arrows or page keys and using the TAB-Key (You can also click the mouse button once). If you press the ENTER key (or doubleclicking the muse button), the Instruction-Information-Windowh_Instruction_information_window is displayed. There are menu commands to set or delete breakpoints.

The Code Menu:

This menu contains the following commands:

+ From Addressh_From_Address

+ Set Breakpointh_Set_Breakpoint

+ Delete Breakpointh_Delete_Breakpoint

## From Address

The beginning address of the code to be displayed in the Code window can be entered via a dialog box. The address specification can be any integer expression (operators and symbols are allowed).

## Set Breakpoint

If you activate this command, a dialog box is displayed to set a breakpoint for a selected instruction in the Code window. See Breakpoint windowh_Breakpointwindow for more details.

## Delete Breakpoint

The breakpoint for a selected instruction in the Code window will be deleted.

## The Pipeline Window

The Structure of the DLX-Pipelineh_Pipelinestructure will be displayed in this Child window. If the window is large enough, additional information about the instructions currently being executed in the pipeline will be displayed.

Each stage is represented by different colours, characterizing this stage through all windows.

After pressing TAB, you can select a stage with the arrow keys. Pressing ENTER (or doubleclicking the stage) allows you to get detailed information about the instruction in the stage via the Instruction-Information-Windowh_Instruction_information_window.

The Pipeline Menu:

+ Display Floating point stagesh_Display_Floatingpointstages

## Display Floating Point Stages

If this option is activated, the floating point stages are displayed in the Pipeline window.

## The Clock Cycle Diagram

In this Child window is displayed, what is done in which cycle and in which stage. Each column represents the state of the pipeline in a given clock cycle. The current state of the pipeline (the last column) is underlayed with gray.

You can select an instruction the same way as in the Code window and get more information about it via the Instruction-Information-Windowh_Instruction_information_window.

Furthermore, details about the stalls and executed forwardings (if enabled) are displayed:

Stalls and Forwarding:

Stalls are represented in boxes, that are underlayed with a colour corresponding to the stalled stage. The contents of the box give more information about the type of the stall:

"R-Stall" - RAW-Stallh_Data_hazards. A dark red arrow shows (if enabled) the instruction that causes the stall.

"T-Stall" - Trap-Stall. This stall occurs only by a Trap-Instructionh_Trap_Instructions . A Trap instruction remains in the IF-stage, until no other instructions are in the pipeline.

"W-Stall" - WAW-Stallsh_Data_hazards. A dark red arrow shows (if enabled) the instruction that causes the stall.

"S-Stall" - structural Stallh_Structural_hazards, There are not enough resources to serving the instruction.

"Stall" - If a Floatingpoint instruction is in MEM-stage, the next instruction will be stalled in the intEX-stage, labeled with "Stall".

If "Display Forwarding" is enabled, forwarding is represented with a green arrow, showing the soucre and the destination of forwarding.

See also Instruction executionh_Instruction_execution for more details.

The Clock Cycle Diagram Menu:

The clock cycle diagram menu contains the following options:

+ Display Forwardingh_Display_Forwarding

+ Display Cause of Stallsh_Display_Stallcause

+ Delete Historyh_Delete_History

+ Set History Lengthh_Set_Historylength

## Display Forwarding

If this option is selected, the source and the destination of forwarding is marked with a green arrow in the clock cycle diagram.

## Display Cause of Stalls

If this option is selected, the instruction that causes a RAW-Stall ("R-Stall") or WAW-Stalls ("W-Stall") is marked with an arrow.

## Delete History

If you activate this command, the history will be deleted. The terminated instructions can not be displayed anymore in the clock cycle diagram.

## Set History Length

The History contains information about terminated instructions. You can specify the length of history (it must be between 0 and 100). A history length of zero means that only the instructions currently being executed in the pipeline are displayed in the clock cycle diagram.

## The Statistics Window

Statistics about the simulation are displayed in the Statistics window. This data is arranged in several groups. You can select the groups to be displayed in the Statistics window. The group "Total" will always be displayed and contains:

+ Total number of cycles executed.

+ The number of instructions, which have already passed the ID-stage.

+ The number of instructions currently being executed in the pipeline (if Detail infoh_Detailinfo (see Statistic Menu) is set).

The Statistic Menu:

This menu contains several options to control  the display of information in the Statistics window. Further, you can reset the statistics:

+ Display:

- Hardwareh_Display_Hardware

- Stallsh_Display_Stalls

- Conditional Branchesh_Display_conditional_branches

- Load/Store-Instructionsh_Display_LoadStore_Instructions

- Floating point stages instructionsh_Display_Floatingpointstage_instructions

- Trapsh_Display_Traps

- Allh_Display_All

+ Detail infoh_Detailinfo

+ Reseth_Reset

## Display Hardware

The following pieces of information are displayed about the hardware configuration of DLX:

+ Size of memory (bytes).

+ Number and delays of floating point units faddEX, fmulEX und fdivEX.

+ Status of Forwarding-Mechanism (enabled or not).

The option Detail info makes no sense to this group!

## Display Stalls

If this option is activated, the following statistics are displayed (absolute and relative):

+ Number of the RAW-Stalls. If forwarding is enabled and the option Detail info is activated, the number of stalls will be splitted in:

- Number of RAW-Stalls with a load instruction causing the stall.

- Number of RAW-Stalls with a branch/jump instruction causing the stall.

- Number of RAW-Stalls with a floatingpoint instruction causing the stall.

+ Number of the WAW-Stalls

+ Number of the structural stalls before floating point stages-instructions

+ Number of the Control-Stalls (= number of taken conditional branches).

+ Number of stalls with a trap instruction causing the stall.

See Instruction executionh_Instruction_execution for more details about stalls.

## Display Conditional Branches

Following informations about conditional branches are displayed (absolute and relative):

+ Number of conditional branches. If the option Detailinfo is activated, the information is splitted:

- Number of taken branches

- Number of not taken branches

## Display Load-/Store-Instructions

If this option is activated, following pieces of information are displayed:

+ Number of executed loads and stores. If the option Detail info is activated, this information will be splitted:

- Number of load instructions

- Number of store instructions

## Display Floating Point Stage Instructions

If this option is activated, you can get information about the number of instructions executed using the floating point stages (faddEX, fmulEX oder fdivEX):

+ Total number of these instructions. Is the option Detail info activated, the information will be splitted:

- Number of instructions that passed the faddEX-Stage.

- Number of instructions that passed the fmulEX-Stage.

- Number of instructions that passed the fdivEX-Stage.

## Display Traps

The total number of executed traps is displayed.

## Display All

All groups will be displayed.

## Detail Info

This option allows you to get detailed information about some groups in the Statistics window.

## Reset

The statistics will be reset. The last simulated clock cycle will be the cycle 0. The history and the pipeline contents will remain unchanged. Statistics will be reset too after a processor reset.

## The Breakpoint Window

In the Breakpoint window, you can view, set, change or delete breakpoints. It is possible to set up to 20 breakpoints. You can also set /delete breakpoints via the Code window.

The Breakpoint Menu:

This menu allows you to manipulate breakpoints. To do this, you must first select a breakpoint using arrow keys or the mouse. Doubleclicking a breakpoint allows to change the breakpoint.

The Breakpoint menu contains the following commands:

+ Seth_Set

+ Deleteh_Delete

+ Delete Allh_Delete_All

+ Changeh_BChange

## Set

After activating this command, a dialog box is displayed, that allows you to set a breakpoint specifying:

+ the address can be an arbitrary integer expression (including operators and symbols). The adress must be a multiple of 4 (aligned at word boundary). Otherwise, the address is converted to the next multiple of 4.

+ the Type can be IF, ID, EX, MEM und WB, which means that the simulation is aborted, if the specified instruction reaches the pipeline stage specified by type. A Read-Breakpoint is reached, when a read access (e.g. load instruction or instruction fetch) to the word (or a part of it) specified by its address is performed. A Write-Breakpoint is reached, when a write access (e.g. store instruction or certain traps) to the word (or a part of it) specified by its address is performed.

The entered values are default values for the next time when this commandis activated.

## Delete

A selected breakpoint can be deleted.

## Delete All

All breakpoints can be deleted with a single command. You will be asked to confirm the action.

## Change

The address or the type of a selected breakpoint can be changed within a dialog window.

## The Memory Windows

You can open up to 10 memory windows with the command memory / Display in the main menu.

For more details about memory windows see also Memory / Displayh_Display.

The Memory Display Menu

This menu contains the following commands, that are applicable to the active memory window:

+ Changeh_MW_Change

+ Closeh_MW_close

## Change

A dialog box is displayed, allowing you to change the configuration of the active memory window. See also Memory / Displayh_Display.

## Close

The active memory window will be closed.

## The DLX-Standard-I/O-Window

This window will be displayed, when:

+ The user has activated the Execution / Display DLX-I/Oh_Display_DLX_IO in the Main menu.

+ A trap is executed in a single step and the standard-(error-)-output is not redirected to a file.

+ A trap is executed in a single step and an input is required and the standard-input is not redirected to a file. You can enter a string terminated by ENTER.

+ More than one step is executed (e.g. Execution / Run toh_Run_to). This allows you to abort a simulation pressing the Cancel-button. If a trap is executed that requires an input, a message is diplayed at the top of the DLX-I/O-Window. During the simulation, the elapsed time for the simulation is displayed.

The DLX-I/O-System Menu:

This window is the only dialog window that has an extended system menu, containing the following additional options:

DLX-Standard-I/O / Delete Window

The DLX-I/O-window is cleared. The cursor is placed at the left corner on the top of the window.

DLX-Standard-I/O / Redirect to File

This option has a submenu:

+ Standard input

+ Standard output

+ Standard error

You can redirect any of these in/outputs to a file via a dialog window. The default extensions for the filenames are:

+ "IN"for the standard input

+ "OUT"for the standard output

+ "ERR" for the standard error

After resetting the processor, all redirections are canceled.

DLX-Standard-I/O / Redirect to Window

This option has a submenu:

+ Standard input

+ Standard output

+ Standard error

An active redirection to a file can be canceled.

## The Instruction-Information Window

This window displays, when an instruction is doubleclicked in the code windowh_Codewindow, clock cycle diagramh_Clockcycle_diagram or pipeline windowh_Pipelinewindow.

You can get detailed information about the simulation of an instruction. There are sections within this window. The left section in the first raw contains general information about the instruction and the state of its execution. In the remaining five sections, details about the five steps of the execution (IF, ID, EX, MEM und WB) are displayed.

The General Section:

Following data are displayed:

+ the disassembled instruction

+ its address

+ the instruction in hexadecimal notation

+ the state of execution (in which pipeline stage it is, the termination status (normal,error,aborted))

+ the cycle number, when the execution has begun

+ the number of the last clock cycle of execution

+ how many cycles the execution has taken

Data about each execution step:

+ the beginning and the duration of the step: "Begin (Duration )"

+ the state of execution

+ what is done

+ stalls and the instruction causing them

+ Source and destination of forwardings and the registers bypassed.

See also Instruction executionh_Instruction_execution.

## The Keyboard

Help

F1	Context sensitive help (in menus, dialog windows or in a child window).

Shortcuts in the main menu

CTRL-R	Reset DLX

CTRL-I	Reset all

F3	Load code or data

ALT-F4	Quit WINDLX

CTRL-F6 or TAB	Next child window

SHIFT-F4	Arrange child windows tiled

SHIFT-F5	Arrange child windows cascaded

CTRL-F4	Close a memory window

CTRL-1	Activate register window

CTRL-2	Activate code window

CTRL-3	Activate pipeline window

CTRL-4	Activate clock cycle diagram

CTRL-5	Activate statistics window

CTRL-6	Activate breakpoint window

CTRL-7	Activate memory window-1

CTRL-8	Activate memory window-2

CTRL-9	Activate memory window-3

CTRL-0	Display list box to activate child window

F7	Execute single cycle

F8	Execute multiple cycles

F5	Run

F4	Run to...

CTRL-O	Display DLX-I/O window

F6	Open memory window

F2	Change memory content

CTRL-Y	View, change, add or delete symbols

CTRL-L	Load configuration

CTRL-S	Save configuration

In register window

Arrows	Select a register

ENTER	Change register content

In code window

TAB	Enter selection mode

ESC	Exit selection mode

Arrows	Select in selection mode, otherwise scroll up/down

ENTER	Display instruction-information window for a selected instruction

INSERT	Set breakpoint for selected instruction

DELETE	Delete breakpoint for selected instruction

In pipeline window

TAB	Enter selection mode

ESC	Exit selection mode

Arrows	Select instruction

ENTER	Display instruction-iInformation window

In clock cycle diagram

TAB	Enter selection mode

ESC	Exit selection mode

Arrows	Select in selection mode, otherwise scroll up/down

ENTER	Display instruction-information window for a selected instruction

In statistics window

Arrows	Scroll up/down

In breakpoint window

TAB	Enter selection mode

ESC	Exit selection mode

Arrows	Select in selection mode, otherwise scroll up/down

ENTER	Change selected breakpoint

INSERT	Set new breakpoint

DELETE	Delete selected breakpoint

## DLX-Examples

Following three examples are described:

+ Greatest common measureh_GCM

+ Factorialh_FACT

+ Prime number tableh_PRIM

## Greatest common measure

This example shows the usage of two modules linked together. Two numbers are read from standard input and the gretest common measure of these two numbers is displayed on standard output.

The Listing (GCM.S)

;*********** WINDLX Bsp. 1: Greatest common measure *************

;*********** (c) 1991 Günther Raidl *************

;------------------------------------------------------------------------

; requires module INPUT.

; Program begins at symbol main

; Read two positive integer numbers from stdin, calculate the gcm

; and write the result to stdout

;------------------------------------------------------------------------

.data

;*** Prompts for input

Prompt1:	.asciiz	"First Number: "

Prompt2:	.asciiz	"Second Number: "

;*** Data for printf-Trap

PrintfFormat:	.asciiz	"gcM=%d\n\n"

.align	2

PrintfPar:	.word	PrintfFormat

PrintfValue:	.space	4

.text

.global	main

main:

;*** Read two integer positive numbers into R1 and R2

addi	r1,r0,Prompt1

jal	InputUnsigned	;read uns.-Integer into R1

add	r2,r1,r0	;R2 <- R1

addi	r1,r0,Prompt2

jal	InputUnsigned	;read uns.-Integer into R1

Loop:	;*** Compare R1 and R2

seq	r3,r1,r2	;R1 == R2 ?

bnez	r3,Result

sgt	r3,r1,r2	;R1 > R2 ?

bnez	r3,r1Greater

r2Greater:	;*** subtract r1from r2

sub	r2,r2,r1

j	Loop

r1Greater:	;*** subtract r2 from r1

sub	r1,r1,r2

j	Loop

Result:	;*** Write the result (R1)

sw	PrintfValue,r1

addi	r14,r0,PrintfPar

trap	5

;*** end

trap	0

The Listing of INPUT.S

;*********** WINDLX Ex. 1: Read a positive integer value *************

;*********** (c) 1991 Günther Raidl *************

;-----------------------------------------------------------------------------

;Subprogram call by symbol "InputUnsigned"

;expect the address of a zero-terminated prompt string in R1

;returns the read value in R1.

;changes the contents of registers R1, R13, R14

;-----------------------------------------------------------------------------

.data

;*** Data for Read-Trap

ReadBuffer:	.space	80

ReadPar:	.word	0,ReadBuffer,80

;*** Data for Printf-Trap

PrintfPar:	.space	4

SaveR2:	.space	4

SaveR3:	.space	4

SaveR4:	.space	4

SaveR5:	.space	4

.text

.global	InputUnsigned

InputUnsigned:

;*** save register contents

sw	SaveR2,r2

sw	SaveR3,r3

sw	SaveR4,r4

sw	SaveR5,r5

;*** prompt

sw	PrintfPar,r1

addi	r14,r0,PrintfPar

trap	5

;*** call Trap-3 to read line

addi	r14,r0,ReadPar

trap	3

;*** determine value

addi	r2,r0,ReadBuffer

addi	r1,r0,0

addi	r4,r0,10	;Dezimalsystem

Loop:	;*** read digits to end of line

lbu	r3,0(r2)

seqi	r5,r3,10	;LF -> Exit

bnez	r5,Finish

subi	r3,r3,48	;´0´

multu	r1,r1,r4	;Shift decimal

add	r1,r1,r3	;

addi	r2,r2,1	;increment pointer.

j	Loop

Finish:		;*** restore old registers

lw	r2,SaveR2

lw	r3,SaveR3

lw	r4,SaveR4

lw	r5,SaveR5

jr	r31	; Return

This program contains no error handling for bad inputs.

## Factorial

This example shows the usage of floating point instructions. The module INPUT (defined in the previous example) is also used to read an integer value.

The Listing of Factoriel (FACT.S)

;*********** WINDLX Ex. 2: Factorial *************

;*********** (c) 1991 Günther Raidl *************

;--------------------------------------------------------------------------

; Program begins at symbol main

; requires module INPUT

; reads a number from stdin and calculates the factorial (Type: double)

; the result is written to stdout

;--------------------------------------------------------------------------

.data

Prompt:	.asciiz	"An integer value >1 : "

PrintfFormat:	.asciiz	"Factorial = %g\n\n"

.align	2

PrintfPar:	.word	PrintfFormat

PrintfValue:	.space	8

.text

.global	main

main:

;*** Read value from stdin into R1

addi	r1,r0,Prompt

jal	InputUnsigned

;*** init values

movi2fp	f10,r1	;R1 -> D0, D0..Count register

cvti2d	f0,f10

addi	r2,r0,1	;1 -> D2, D2..result

movi2fp	f11,r2

cvti2d	f2,f11

movd	f4,f2	;1-> D4, D4..Constant 1

;*** Break loop, if D0 = 1

Loop:	led	f0,f4	;D0<=1 ?

bfpt	Finish

;*** multiplication and next loop

multd	f2,f2,f0

subd	f0,f0,f4

j	Loop

Finish:	;*** write result to stdout

sd	PrintfValue,f2

addi	r14,r0,PrintfPar

trap	5

;*** end

trap	0

## Prime Number Table

The Listing of Prime number table generator (PRIM.S)

;*********** WINDLX Ex. 3: Generate Prime number table *************

;*********** (c) 1991 Günther Raidl *************

;-------------------------------------------------------------------

; Program begins at symbol main

; generates a table with the first ´Count´ prime numbers from ´Table´

;-------------------------------------------------------------------

.data

;*** Size of table

.global	Count

Count:	.word	10

.global	Table

Table:	.space	Count*4

.text

.global	main

main:

;*** Initialization

addi	r1,r0,0	;Index in Table

addi	r2,r0,2	;Current value

;*** Determine, if R2 can be divided by a value in table

NextValue:	addi	r3,r0,0	;Helpindex in Table

Loop:	seq	r4,r1,r3	;end of table?

bnez	r4,IsPrim	; R2 is a prime number

lw	r5,Table(R3)

divu	r6,r2,r5

multu	r7,r6,r5

subu	r8,r2,r7

beqz	r8,IsNoPrim

addi	r3,r3,4

j	Loop

IsPrim:	;*** Write value in table and increment index

sw	Table(r1),r2

addi	r1,r1,4

;*** 'count' reached?

lw	r9,Count

srli	r10,r1,2

sge	r11,r10,r9

bnez	r11,Finish

IsNoPrim:	;*** Check next value

addi	r2,r2,1	;increment R2

j	NextValue

Finish:	;*** end

trap	0

## Structure of the Pipeline

The processor "DLX" (read: DeLuXe) has a pipeline with five pipe stages:

+ IF-Stageh_IF_Stage

+ ID-Stageh_ID_Stage

+ EX-Stages:h_EX_Stage

- intEX-Stageh_intEX_Stage

- faddEX-Stage(s)h_faddEX_Stage

- fmulEX-Stage(s)h_fmulEX_Stage

- fdivEX-Stage(s)h_fdivEX_Stage

+ MEM-Stageh_MEM_Stage

+ WB-Stageh_WB_Stage

## IF-Stage (Instruction Fetch)

On each clock cycle another instruction is fetched from memory into the instruction register. The PCh_PC is incremented to point to the next instruction.

## ID-Stage (Instruction decoding)

The instruction is decoded and the register file is accessed to read the operand registers Ah_A_B and Bh_A_B. Branches are already executed in this stage to reduce the control stalls.

## EX-Stages (EXecution)

In this stage, the proper unit is selected to operate on the operands prepared in the prior step. The execution stage contains units for integer operations (intEXh_intEX_Stage exists once) and for floating point operations: faddEXh_faddEX_Stage, fmulEXh_fmulEX_Stage and fdivEXh_fdivEX_Stage (multiple units possible).

## intEX-Stage (INTeger EXecution):

This unit performs integer arithmetic operations (except multiply and divide) and effective address calculation for memory references and branches.

## faddEX-Stage (Floating point ADDition EXecution):

This unit performs floating point addition and subtraction of single- or double precision floating point numbers.

## fmulEX-Stage (Floating Point MULtiplication EXecution):

This unit performs multiply operations of single or double precision floating point numbers but also on integer numbers (signed and unsigned).

## fdivEX-Stage (Floating Point DIVision EXecution):

This unit performs divide operations of single or double precision floating point numbers but also on integer numbers (signed and unsigned).

## MEM-Stage (MEMory access)

The only DLX instructions active in this step are loads and stores. If the instruction is a load, data are read from memory; if it is a store, then the data are written into memory. In either case the address used is the one computed during the prior step.

## WB-Stage (Write Back)

The result is written into the register file, either coming from the memory system or from the ALU. The operations in this stage are performed during the first half of the clock cycle, so the instruction executing the ID-stage can read the results in the second half of the clock cycle, eliminating the need to bypass the reult to this instruction.

## Register Set

DLX has the following registers, that are visible to programs:

+ GPRs (R0..R31)h_GPRs

+ FPRs (F0..F31)h_FPRs

+ FPSRh_FPSR

+ PCh_PC

DLX has also internal registers, that are not visible to DLX-programs:

+ IMARh_IMAR

+ IRh_IR

+ A, Bh_A_B

+ BTAh_BTA

+ ALUh_ALU

+ DMARh_DMAR

+ SDRh_SDR

+ LDRh_LDR

## General Purpose Register (GPRs, Integer Register)

The architecture has thirty-two 32-bit general-purpose registers; the value of R0 is always 0.

## Floating Point Registers (FPRs)

There are a set of Floating point registers, which can be used as 32 single-precision (32-bit) registers (F0 to F31), or as even-odd pairs holding double-precision values (D0 to D30).

## Floating Point Status Register (FPSR)

The FP status register is used both for compares and FP exceptions. All moves to/from the status register go through the GPRs; there is a branch that tests the comparison bit in the FP status register.

## Program Counter (PC)

The program counter always contains the address of the next instruction to be fetched. Branches and jumps can change its contents.

## Instruction Memory Address Register (IMAR)

This register is initialized with the content of the program counter in the IF stage, because it has a connection to the memory system, but PC doesn't.

## Instruction-Register (IR)

In the IF-stage, this register is loaded with the next instruction to be executed.

## ALU Operand Registers A and B

In the ID-stage, the two source registers are read and their value is sent to the ALU operand registers A and B. In WinDLX, there also exist the (Pseudo-)registers AHI und BHI, that contain the upper 32 bits for double-precision Floating point values.

## Branch Target Address Register (BTA)

In the ID-stage, the branch/jump target address is computed and written into this register. If the branch is taken, this value is loaded to the program counter.

## ALU Output Register (ALU)

The result of an ALU operation is transferred to this register. In WinDLX, there exists the (pseudo-)register ALUHI, that contains the higher 32 bits for double-precision Floating point values.

## Data Memory Address Register (DMAR)

The address for a memory reference is transferred to this register. In the MEM-stage, the memory reference is performed with this register.

## Store Data Register (SDR)

For a store instruction, the data to be stored into the memory is transferred to this register. In WinDLX, there exists the (pseudo-)register SDRHI, that contains the higher 32 bits of double-precision Floating point values.

## Load Data Register (LDR)

For a load instruction, the data is read from memory into this register. In WinDLX, there exists the (pseudo-)register LDRHI, that contains the higher 32 bits of double-precision Floating point values.

## Instruction Set of DLX

There are four classes of instructions:

+ Data transfer instructionsh_Datatransfer_instructions

+ Integer arithmetic and logical instructionsh_integer_instructions

+ Control instructionsh_Control_instructions

+ Floating point instructionsh_Floatingpoint_instructions

All instructions are 32 bits wide with a 6-bit primary opcode. Basically, there are three different instruction types:

I-type instructions:

Encodes:

+ Loads- and Stores (rd <- Mem[rs1+Immediate])

+ All immediates (rd <- rs1 op Immediate)

+ Conditional branch instructions (rs1 is register, rd unused)

+ JR (jump register) and JALR (jump and link register), where rd=0, rs1=Register, Immediate=0

R-type instructions:

+ Register-Register ALU operations rd <- rs1 func rs2:

+ MOVxxx-Instructions (rd <- rs1)

J-type instructions:

Encodes:

+ J (jump) and JAL (jump and link)

+ TRAP

+ RFE (return from exception)

## Instructions for Data Transfer

Move data between registers and memory, or between integer and FP or special registers; only memory address mode is 16-bit displacement + contents of a GPR:

LB Rd,Adr	Load byte (sign extension)

LBU Rd,Adr	Load byte (unsigned)

LH Rd,Adr	Load halfword (sign extension)

LHU Rd,Adr	Load halfword (unsigned)

LW Rd,Adr	Load word

LF Fd,Adr	Load single-precision Floating point

LD Dd,Adr	Load double-precision Floating point

SB Adr,Rs	Store byte

SH Adr,Rs	Store halfword

SW Adr,Rs	Store word

SF Adr,Fs	Store single-precision Floating point

SD Adr,Fs	Store double-precision Floating point

MOVI2FP Fd,Rs	Move 32 bits from integer registers to FP registers

MOVI2FP Rd,Fs	Move 32 bits from FP registers to integer registers

MOVF Fd,Fs	Copy one Floating point register to another register

MOVD Dd,Ds	Copy a double-precision pair to another pair

MOVI2S SR,Rs	Copy a register to a special register (not implemented!)

MOVS2I Rs,SR	Copy a special register to a GPR (not implemented!)

## Integer arithmetic and logical instructions

Operations on integer or logical data in GPRs; overflows by signed arithmetics are not reported.

ADD Rd,Ra,Rb	Add

ADDI Rd,Ra,Imm	Add immediate (all immediates are 16 bits)

ADDU Rd,Ra,Rb	Add unsigned

ADDUI Rd,Ra,Imm	Add unsigned immediate

SUB Rd,Ra,Rb	Subtract

SUBI Rd,Ra,Imm	Subtract immediate

SUBU Rd,Ra,Rb	Subtract unsigned

SUBUI Rd,Ra,Imm	Subtract unsigned immediate

MULT Rd,Ra,Rb	Multiply signed

MULTU Rd,Ra,Rb	Multiply unsigned

DIV Rd,Ra,Rb	Divide signed

DIVU Rd,Ra,Rb	Divide unsigned

AND Rd,Ra,Rb	And

ANDI Rd,Ra,Imm	And immediate

OR Rd,Ra,Rb	Or

ORI Rd,Ra,Imm	Or immediate

XOR Rd,Ra,Rb	Xor

XORI Rd,Ra,Imm	Xor immediate

LHI Rd,Imm	LOad high immediate - loads upper half of register with immediate

SLL Rd,Rs,Rc	Shift left logical

SRL Rd,Rs,Rc	Shift right logical

SRA Rd,Rs,Rc	Shift right arithmetic

SLLI Rd,Rs,Imm	Shift left logical 'immediate' bits

SRLI Rd,Rs,Imm	Shift right logical 'immediate' bits

SRAI Rd,Rs,Imm	Shift right arithmetic 'immediate' bits

S__ Rd,Ra,Rb	Set conditional: "__" may be EQ, NE, LT, GT, LE or GE

S__I Rd,Ra,Imm	Set conditional immediate: "__" may be EQ, NE, LT, GT, LE or GE

S__U Rd,Ra,Rb	Set conditional unsigned: "__" may be EQ, NE, LT, GT, LE or GE

S__UI Rd,Ra,Imm	Set conditional unsigned immediate: "__" may be EQ, NE, LT, GT, LE or GE

NOP	No operation

## Control instructions

Control is handled through a set of jumps and a set of branches. the four jump instructions are differentiated by the two ways of specifying the destination address and by wether or not a link is made. Two jumps use a 26-bit offset added to the program counter to determine the destination address; the other two specify a register that is the destination address. There are two flavors of jumps: plain jump, and jump and link (used for procedure call). The latter places the return address in R31. All branches are conditional. The branch condition is specified by the instruction, which may test the register source for zero or nonzero; this may be a data value or the result of a compare. The branch target address is specified with a 16-bit signed offset that is added to the program counter. In WINDLX, the jumps and the branches are completed by the end of the ID cycle to reduce the number of control stalls. In the DLX pipeline, the predict-not-taken scheme is implemented by continuing to fetch instructions as if the branch were a normal instruction. The pipeline looks as if nothing out of the ordinary is happening. If the branch is taken, we neeed to stop the pipeline and restart the fetch.

BEQZ Rt,Dest	Branch if GPR equal to zero; 16-bit offset from PC

BNEZ Rt,Dest	Branch if GPR not equal to zero; 16-bit offset from PC

BFPT Dest	Test comparison bit in the FP status register (true) and branch; 16-bit offset from PC

BFPF Dest	Test comparison bit in the FP status register (false) and branch; 16-bit offset from PC

J Dest	Jump: 26-bit offset from PC

JR Rx	Jump: target in register

JAL Dest	Jump and link: save PC+4 to R31; target is PC-relative

JALR Rx	Jump and link: save PC+4 to R31; target is a register

TRAP Imm	Transfer to operating system at a vectored address; see Trapsh_Traps.

RFE Dest	Return to user code from an execption; restore user mode (not implemented!)

## Floating Point Instructions

Floating point instructions manipulate the floating point registers and indicate whether the operation to be performed is single or double precision. Single-precision operations can be applied to any of the registers, while double-precision operations apply only to even-odd pair (e.g. F4,F5), which is designated by the even register number.

ADDD Dd,Da,Db	Add double-precision numbers

ADDF Fd,Fa,Fb	Add single-precision numbers

SUBD Dd,Da,Db	Subtract double-precision numbers

SUBF Fd,Fa,Fb	Subtract single-precision numbers.

MULTD Dd,Da,Db	Multiply double-precision Floating point numbers

MULTF Fd,Fa,Fb	Multiply single-precision Floating point numbers

DIVD Dd,Da,Db	Divide double-precision Floating point numbers

DIVF Fd,Fa,Fb	Divide single-precision Floating point numbers

CVTF2D Dd,Fs	Converts from type single-precision to type double-precision

CVTD2F Fd,Ds	Converts from type double-precision to type single-precision

CVTF2I Fd,Fs	Converts from type single-precision to type integer

CVTI2F Fd,Fs	Converts from type integer to type single-precision

CVTD2I Fd,Ds	Converts from type double-precision to type integer

CVTI2D Dd,Fs	Converts from type integer to type double-precision

__D Da,Db	Double-precision compares: "__" may be EQ, NE, LT, GT, LE or GE; sets comparison bit in FP status register

__F Fa,Fb	Single-precision compares: "__" may be EQ, NE, LT, GT, LE or GE; sets comparison bit in FP status register

## Traps - the System Interface

Traps build the interface between DLX programs and the I/O-system. There are five traps defined in WinDLX. Zero is an invalid parameter for a trap instruction, used to terminate a program.

The Traps:

+ Trap #1: Open Fileh_Trap1

+ Trap #2: Close Fileh_Trap2

+ Trap #3: Read Block from Fileh_Trap3

+ Trap #4: Write Block to Fileh_Trap4

+ Trap #5: Formatted Output to Standard-Outputh_Trap5

For all five defined traps:

+	They match the UNIX/DOS-System calls resp. C-library-functions open(), close(), read(), write() and printf().

+	The file descriptors 0,1 and 2 are reserved for the stdin, stdout and stderr. The input and output to the DLX-I/O-Windows can be controlled with this descriptors.

+	The address of the required parameters for the system calls must be loaded in register R14.

+	All parameters have to be 32 bits long (except the double-precision FPs that are 64 bits long). Strings are referenced with their begin address.

+	The result is returned into R1.

+	If an error occurs during the execution of a system call, register R1 is set to -1 and: if the symbol "_errno" is set to value A, then an error code is returned at the memory address A and the simulation continues, otherwise the simulation is aborted. See MS-DOS documentation for details about the error codes.

## Trap #1: Open File

You can open a file for reading or to writing. Open files are automatically closed after a DLX reset or when quitting the WinDLX.

Parameter:

1. filename: Address of a zero-terminated string, that contains the path of the file to be opened.

2. mode, to open the file. Following modes can be combined using logical or-operation:

0x0001		O_RDONLY	(read only)

0x0002		O_WRONLY	(write only)

0x0004		O_RDWR	(read and write)

0x0100		O_CREATE	(create file)

0x0200		O_TRUNC	(truncate file)

0x0400		O_EXCL	(open file exclusively (with SHARE))

0x0800		O_APPEND	(append to file)

0x4000		O_TEXT	(Convert CR/LF)

0x8000		O_BINARY	(No conversion of CR/LF)

(Warning: These flags are predefined in MS-DOS and not compatible to UNIX.)

3. Additional Flags:

0x0000		S_IFREG	(Normal file, no directory etc.)

0x0100		S_IREAD	(read access permission)

0x0080		S_IWRITE	(write access permission)

0x0040		S_IEXEC	(execute permission)

(Warning: These flags are predefined in MS-DOS and not compatible to UNIX.)

(See C-Library functions for more details.)

The file descriptor is returned in register R1.

Example:

.data

FileName:	.asciiz	"c:\\BSP\\DATEI.DAT"

.align	2

Par:	;*** Parameters for Trap1 (OPEN)

.word	FileName

;create for R/W:

.word	0x0104

;R/W-access permission:

.word	0x0180

FileDescr:	.space	4

.text

lhi	r14,Par>>16		; upper 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	1

sw	FileDescr,R1

## Trap #2: Close File

A file previously opened with Trap #1 is closed.

Parameter:

1. File descriptor of the file to be closed.

Zero is returned in register R1, if it was successful, -1 otherwise.

Example:

.data

FileDescr:	.space	4

.text

;...

lhi	r14,FileDescr>>16		; higher 16 bits

addui	r14,r14,FileDescr&0xffff	; lower 16 bits

trap	2

## Trap #3: Read Block from File

A file block or a line from stdin can be read with this trap.

Parameter:

1. File descriptor of the file

2. Address, for the destination of the read operation

3. Size of block (bytes) to be read

The actual number of bytes read is returned in register R1.

Example:

.data

Buffer:	.space	500

Par:	;File descriptor, Adr. of buffer, size of block

.space	4

.word	Buffer

.word	500

.text

;...

lhi	r14,Par>>16		; higher 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	3

## Trap #4: Write Block to File

A block can be written to the memory or the standard output.

Parameter:

1. File descriptor of file

2. Address of block to be written

3. Size of the block (bytes)

The actual number of bytes written is returned in register R1.

Example:

.data

Buffer:	.space	500

Par:	;File descriptor, Adr. of buffer, size of block

.space	4

.word	Buffer

.word	500

.text

;...

lhi	r14,Par>>16		; higher 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	4

## Trap #5: Formatted Output to Standard Output

This function is equivalent to the C-library function printf().

Parameter:

1. Format string see C-function printf()

2.... Arguments according to format string; see C-function printf()

The number of bytes transferred to stdout is returned in register R1.

Example:

.data

FormatStr:	.asciiz	"Pi=%f, N=%d\n"

.align	2

Par:	;Adr. of formatstring, values

.word	FormatStr

.double	3.141592654

.word	17

.text

;...

lhi	r14,Par>>16		; higher 16 bits

addui	r14,r14,Par&0xffff	; lower 16 bits

trap	5

## The DLX-Assembly language

In WinDLX, you can load one or more DLX-source files. The files are loaded in alphabetical order.. In addition to the DLX-Instructionsh_Instructions , the assembler supports several directives which affect how it fills the memory.

+ Syntax for Expressionsh_Syntax

+ Directivesh_directives

## Syntax for Expressions

The syntax for expressions is C-like (e.g. strings must be quoted with " ").

Numbers: WinDLX accepts numbers in decimal notation, hexadecimal notation if the first two characters of the number are 0x, or octal notation if the first character is 0.

Address expressions: The simplest form of such an expression is a number, which is interpreted as a memory address. More generally, address expressions may consist of numbers, symbols (which must be defined in the assembly files currently loaded) , the operators *, /, +, -, <<, >>, &, |, and ^ (which have the same meanings and precedences as in C), and parentheses for grouping.

The value of an expression must be in the defined range proper to the type of the expression:

Bytes:		-128 .. +255 (8 Bit)

Halfwords:	-32768 .. +65535 (16-Bit)

Words:		-2147483648 ..+2147483647 oder 0xffffffff (32-Bit)

## Directives

While the assembler is processing an assembly file, the data and instructions it assembles are placed in memory based on either a text (code) or data pointer. Which pointer is used is not selected by the type of information, but whether the most recent directive was .data or .text. The program initially loads into the text segment. By default, the code is loaded at location $CODE (initially set to 0x100) and the data are loaded at location $DATA (initially set to 0x1000).

.align n	Cause the next data/code loaded to be at the next higher address with the lower n bits zeroed (the next closest address greater than or equal to the current address that is a multiple of 2n (e.g. .align 2 means the next word begin)

.ascii "string1","..."	Store the "strings" listed on the line in memory as a list of characters. The strings are not terminated by a 0 byte.

.asciiz "string1","..."	Similar to .ascii, except each string is terminated by a 0 byte.

.byte byte1,byte2,...	Store the bytes listed on the line sequentially in memory.

.data [address]	Cause the following code and data to be stored in the data area. If an address was supplied, the data will be loaded starting at that address, otherwise, the last value for the data pointer will be used. If we were just reading data based on the text (code) pointer, store that address so that we can continue from there later (on a .text directive).

.double number1,...	Store the "numbers" listed on the line sequentially in memory as double-precision Floating point numbers.

.global label	Make the label available for reference by code found in files loaded after this file.

.space size	Move the current storage pointer forward size bytes (to leave some empty space in memory)

.text [address]	Cause the following code and data to be stored in the text (code) area. If an address was supplied, the data will be loaded starting at that address, otherwise, the last value for the text pointer will be used. If we were just reading data based on the data pointer, store that address so that we can continue from there later (on a .data directive).

.word word1,word2,...	Store the word" listed on the line sequentially in memory.

## Execution of Instructions

For the DLX instruction set (excluding the Execution of Floating point instructionsh_Floatingpoint_execution), all instructions can be broken into five basic steps: fetch, decode, execute, memory-access, and write-result. This steps match the five stages of the DLX pipeline. The following table shows the events on every pipe stage of the DLX pipeline:

Stage	ALU instruction	Load or store instruction	Branch instruction

IF	IMAR <- PC;	IMAR <- PC;	IMAR <- PC;

IR <- Mem[IMAR]	IR <- Mem[IMAR]	IR <- Mem[IMAR]

PC <- PC + 4	PC <- PC + 4	PC <- PC + 4

ID	A <- Rs1; B <- Rs2;	A <- Rs1; B <- Rs2;	A <- Rs1; B <- Rs2;

BTA <- PC + offset;

if (Rs1 op 0) then PC <- BTA;

EX	ALUoutput<- A op B; or	DMAR <- A + immediate;

ALUoutput <- A op immediate	SDR <- B;

MEM	LDR <- Mem[DMAR]; or

Mem[DMAR] <- SDR

WB	Rd <- ALUoutput	Rd <- LDR

The major hurdle of pipelining - Pipeline Hazards

There are situations, called hazards, that prevent the next instruction in the instruction stream from being executed during its designated clock cycle. Hazards reduce the performance of the ideal speedup gained by pipelining. Hazards in pipelines can make it necessary to stall the pipeline. The impact of hazards in causing stalls can be reduced by the use of forwardingh_forwarding. There are three classes of hazards:

+ Structural hazardsh_Structural_hazards

+ Data hazardsh_Data_hazards

+ Control hazardsh_Control_hazards

In WinDLX, traps can also can cause stalls, see Trap instructionh_Trap_instructions.

## Structural Hazards

Structural hazards arise from resource conflicts when the hardware cannot support all possible combinations of instructions in simultaneous overlapped execution.

## Data Hazards

Data hazards arise when an instruction depends on the results of a previous instruction in a way that is exposed by the overlapping of instructions in the pipeline. Data hazards may be classified as one of three types, depending on the order of read and write accesses in the instructions. Consider two instructions i and j, with i occuring before j. The possible data hazards are:

i) RAW (read after write): j tries to read a source before i writes it, so j incorrectly gets the old value.

ii) WAR-(write after read): j tries to write a destination before it is read by i, so i incorrectly gets the new value. This cannot happen in WinDLX because all reads are early (in ID) and all writes are late (in WB). This hazard occurs when there are some instructions that write results early in the instruction pipeline, and other instructions that read a source after a write of an instruction later in the pipeline (e.g. autoincrement addressing).

iii) WAW (write after write): j tries to write an operand before it is written by i. The writes end up being performed in the wrong order, leaving the value written by i rather than the value written by j in the destination. This hazard is present only in pipelines that write in more than one pipe stage. The DLX pipeline writes a register only in WB and avoids this class of hazards for integer instructions. This hazard can occur with floating point stage instructions, see Execution of Floating Point Instructionsh_Floatingpoint_execution

## Control Hazards

Control hazards arise from the pipelining of branches and other instructions that change the PC. If instruction i is a taken branch, then the PC is normally not changed until the end of ID.

## Forwarding

Forwarding (also called bypassing and sometimes short-circuiting) is a simple hardware technique to reduce the penalties caused by data hazards. This technique works as follows: The ALU result is always fed back to the ALU input latches. If the forwarding hardware detects that the previous ALU operation has written the register corresponding to a source for the current ALU operation, control logic selects the forwarded result as the ALU input rather than the value read from the register file.

## Trap Instructions

In WinDLX, traps are delayed using stalls until no prior instructions are in the pipeline. That is performed to avoid conflict with other instructions.

## Execution of Floating Point Instructions

The following DLX instructions use the Floating point execution stages:

ADDF, ADDD, SUBF, SUBD	faddEX

MULT, MULTU, MULTF, MULTD	fmulEX

DIV, DIVU, DIVF, DIVD	fdivEX

The Floating point execution stages take more than one cycle to perform the operation (The number of FP units and their delay can be configured in WinDLX).

If more than one Floating point instruction terminate their execution stage simultaneously, it is not defined which instruction enters the MEM stage first. If an integer instruction and a Floating point instruction terminate their execution stage simultaneously, the Floating point instruction enters the next pipeline stage first.

WAW hazards (see Data hazardsh_Data_hazards) can occur with the Floating point instructions (do not complete in order). This conflict is detected in WinDLX before the instruction enters the EX stage and causes WAW stalls.

A simple example:

ADDF	F1,F2,F3	IF	ID	EX	EX	MEM	WB

MOVF	F1,F4			IF	ID	Stall	EX	MEM	WB